Suspension

Perhaps the most unique part of our robot is it's independent passive suspension system. The suspension was first conceived in a brain storming session last year to determine a way of dampening vibration and allowing for better ground clearance. The suspension works by having each motor and wheel mounted on an free moving self contained motor nacelle. The nacelles are mounted on axels that act like hinges allowing them to move up and down. The nacelles are then secured in the down position by surgical rubber tooling that acts as the shock absorber. The suspension allows us to drive off the sides of the ramp without damaging the electronics, it acts to dampen the vibration in our accelerometer and gyro, and it allows us to drive smoothly over the fallen kickstands.

Modular chassis

This year our robot is featuring a predesigned modular chassis. The chassis was designed before the season started as a summer project and was dubbed the KR4. Since we did not know what the competition would be, we used experience from past years to developed a chassis that could be adapted to a wide variety of challenges. We first selected the Kraken style of chassis that was first designed by PHI Robotics five years ago. The Kraken chassis has a high degree of ground clearance to allow it to drive over a wide variety of field elements. We then implemented our Nacelle based suspension to give the chassis even further capabilities. We then looked into how to make the Chassis modular. To do this we used the CAD modeling software Google SketchUp. We spent several months and came up with a rail and bracket system that allows us to slide the center support and mounting bars forward and backward to accommodate any subsystem that we might need.

3D printing

This has been the first year that PHI robotics has been able to add 3D printing to a wide array of tools. Late last year, our team was introduced to the architectural design software SketchUp. We then spent some time trying to acquire the Auto Desk design software however our computers failed to run it so we were stuck with using SketchUp. Last summer we spent a considerable amount of time creating our new chassis with our CAD software. We however did not have enough funds to buy our own 3D printer so we went and asked NASA for help. NASA Armstrong's 3D printer lab offered their services as well as some guidance to help get our parts printed. After some consulting with the NASA engineers, we completed our design and had our first 3D printed parts made. After some trial and error, we have refined our 3D designs and now use 3d printed Nacelles and brackets on our chassis. PHI is now the leading authority on 3D printing amongst the FTC teams in our area and we offer assistance to team in need of 3D printed parts.

Carbon fiber shield

This season our robot is sporting new high tech composite shields made of carbon fiber. Initially the shields where going to be made of fiber glass. We started the process before our second competition this year by using our CAD software to create shields that would mold over the sides of our robot to provide protection for the robots electronics. After making the 3D models we recreated them with cardboard to make stand in shield for our Palmdale competition that would later act as molds too be made in our backyard out of fiber glass. At the Palmdale competition we ran into one of our old friends who used to be on an FRC team and graduated a few years ago. While showing him the shields he asked us if we wanted them made out of carbon fiber instead. We asked him if he knew where we could get them made of carbon fiber and he told us that he now works at Scaled Composites manufacturing carbon fiber components for Space Ship One and Space Ship Two. So we gave him the card board molds and two weeks later he came back with our new shields made of carbon fiber. They are a 3K 2X2 weave laid up in a 90, 45, 90, 45 arrangement. The use West system epoxy with a 105a epoxy and a 205 speed hardener combined 1 to 1.