Anti-static techniques

This year we have had a few issues with the NXT shocking itself and killing the robot making it not work. So we have attempted a few solutions: 1 we put a grounding wire on the robot to get rid of the static which did not work, 2 we have tried rubbing down our whole robot with drier sheets that reduce static, 3 we sprayed the belt with soap and water, 4 and installed some Farrell connectors so that the wires do not pull out and cause shortages, 5 lastly when we rewired the robot we separated the power wires and the command wires lastly we attached a surge protector. Currently the robot has not needed other solutions.

Ball sorting tactics

After our first few tournaments we saw how beneficial it was to have a ball sorter, so we developed a ball sorter of our own. Our ball sorting system is a conveyer belt made using non-slip drawer liner. The rollers that move the belt are made out of PVC pipe and supported by aluminum.  The conveyer belt works by pressing the balls against a sheet of plastic. Because of the distance between the belt and the plastic it can only pick up the baseball-sized whiffle balls.